ABS ECU (Anti-lock Braking System Electronic Control Unit)

Internal Architecture of ABS ECU:

1. Microcontroller (Core Brain)

- Automotive-grade MCU (Infineon / NXP / Renesas)

- Runs at real-time speeds

- Executes control algorithms every few milliseconds

2. Input Stage (Sensors Interface)

- Takes signals from:

  - Wheel speed sensors

- Includes:

  - Signal conditioning (filters, amplifiers)

  - ADC or digital capture

3. Processing Unit

- Calculates:

  - Wheel speed

  - Acceleration/deceleration

  - Slip ratio

- Decision logic:

  - Is wheel locking?

  - Should pressure be reduced?

4. Output Driver Stage

- Controls:

  - Solenoid valves

  - Pump motor

- Uses:

  - MOSFET drivers

  - PWM signals

5. Communication Interface

- Talks to other ECUs using:

  - CAN Bus

6. Power Management

- Converts 12V battery to:

  - 5V / 3.3V for MCU

- Includes:

  - Voltage regulators

  - Protection circuits

7. Memory

- Flash → program storage

- EEPROM → fault codes (DTCs)

- RAM → runtime data

Safety & Reliability Features

Redundancy

- Multiple checks for sensor validity

Watchdog Timer

- Resets ECU if software hangs

Fail-safe Mode

- Falls back to normal braking if ECU fails

Self-Diagnostics

- Detects:

  - Sensor faults

  - Valve faults

  - Communication errors

ECU + HCU Integration

In modern vehicles:

- ABS ECU is often combined with:

  - Hydraulic Control Unit (HCU)

Called: ABS Module

Software Inside ABS ECU

- Usually built on:

  - AUTOSAR architecture

- Written in:

  - Embedded C

Tasks include:

- Sensor data acquisition

- Control algorithm execution

- Communication handling

- Fault detection

| Feature | ABS ECU |
| --- | --- |
| Response Time | Extremely fast (ms level) |
| Type | Hard real-time system |
| Criticality | Safety-critical |
| Control Type | Closed-loop control |

Core Data Handled by ABS ECU

1. Wheel Speed Data

• Individual speed of each wheel (FL, FR, RL, RR)
• Measured in RPM or km/h
• High-frequency signal from sensors

Primary input for all ABS decisions

2. Wheel Acceleration / Deceleration

• Rate of change of wheel speed
• Detects sudden drops (wheel about to lock)

Equivalent to “trend analysis” in BMS

3. Slip Ratio Data

• Difference between vehicle speed and wheel speed

• Indicates traction condition
• Optimal slip ≈ 10–20%

Key performance parameter

4. Brake Pressure Data

• Hydraulic pressure in brake lines
• Monitored indirectly or via sensors

Used to understand braking force applied

5. Vehicle Speed Estimation

• Derived from wheel speeds
• Sometimes fused with other ECUs

Reference value for slip calculation

6. Valve Status Data

• State of solenoid valves:

- Open / Closed
• Pressure mode:

- Increase / Hold / Decrease

7. Pump Motor Status

• ON / OFF condition
• Current consumption (in advanced systems)

8. Fault & Diagnostic Data

• Sensor faults
• Signal loss
• Electrical faults
• Stored as Diagnostic Trouble Codes (DTCs)

9. Road Condition Estimation Data

• Derived from slip patterns
• Detects:

- Wet road

- Dry road

- Low friction (gravel/ice)

Not direct sensors — calculated data

10. System Status Data

• ABS active / inactive
• Warning light status
• System health