DESIGN DOCUMENT

Anti-lock Braking System (ABS)

Platform: STM32F407G Discovery Board

# 1. Project Overview

## 1.1 Project Summary

This project implements a simulated multi-ECU vehicle system on a network of STM32 microcontrollers. Eight ECUs are integrated into a shared communication architecture to replicate the behaviour of a real vehicle's electronic backbone. Each team member is responsible for one ECU sub-system. This document covers the ABS (Anti-lock Braking System) ECU implemented on the STM32F407G Discovery board.

The ABS ECU receives wheel speed data from a Hall-effect sensor, computes slip ratio in real time, applies simulated brake pressure modulation, and communicates key parameters to other ECUs over CAN bus. System behaviour is indicated locally via the on-board LEDs and communicated to the Infotainment ECU for display.

## 1.2 System-Level ECU Map

The following nine ECUs form the complete vehicle simulation network. All communicate over a shared CAN bus at 500 kbps.

| # | ECU Name | Responsible For | Platform | CAN Node ID |
| --- | --- | --- | --- | --- |
| 1 | Engine Management ECU | Engine RPM simulation, temperature | STM32F407G | 0x300 |
| 2 | Transmission ECU | Gear selection, torque converter, shift logic | STM32F407G | 0x400 |
| 3 | ABS ECU (This Document) | Wheel speed, slip ratio, brake modulation, distance, temperature | STM32F407G | 0x200 |
| 4 | Body Control Module (BCM) | Lights, wipers, doors, central locking | STM32F407G | 0x400 |
| 5 | Infotainment ECU | Display of speed, RPM, ABS status, temperature | STM32F407G | 0x800 |
| 6 | Battery Management System (BMS) | Battery voltage, SoC, charge control | STM32F407G | 0x500 |
| 7 | Telematics ECU | GPS, over-the-air data, remote monitoring, immobilizer | STM32F407G | 0x700 |
| 8 | Airbag ECU | Crash detection, airbag deployment trigger, seatbelt pretensioner | STM32F407G | 0x100 |

## 1.3 ABS ECU Scope

The ABS ECU is responsible for the following within the integrated system:

- Reading wheel rotational speed from a Hall-effect sensor

- Computing vehicle speed from wheel RPM and known wheel radius (1 inch = 25.4 mm)

- Simulating ABS brake pressure modulation cycles (hold → release → reapply)

- Indicating ABS activation state via on-board RED LED blinking patterns

- Calculating distance travelled during ABS application

- Estimating brake disc-pad temperature based on braking intensity (imaginary model)

- Monitoring engine RPM from Engine ECU (max 8000 RPM) to correlate with wheel speed

- Applying emergency braking command when immobilization signal is received from ADAS ECU

- Receiving crash/impact signal from Airbag ECU and logging it as a DTC freeze-frame fault event

- Transmitting wheel speed, ABS status, temperature, and distance over CAN bus

- Receiving speed override and immobilization commands over CAN bus

# 2. Hardware Platform — STM32F407G Discovery Board

## 2.1 On-Board LED Assignment for ABS Project

The STM32F407G Discovery board has four user-controllable LEDs. Each is assigned a specific role in the ABS system:

| LED | Pin | Colour | ABS System Function | Behaviour |
| --- | --- | --- | --- | --- |
| LD3 | PD13 | Orange | ABS System READY / Power ON | ON solid when system initialized; OFF on fault |
| LD4 | PD12 | Green | Normal Braking Active | ON solid when brake is applied normally (no ABS) |
| LD5 | PD14 | Red | ABS ACTIVE Indicator | Fast blink during ABS; blink rate = modulation intensity |
| LD6 | PD15 | Blue | Immobilizer / Emergency Brake | ON solid when immobilizer command received; all others OFF |

RED LED (LD5) Blink Pattern Logic — ABS Intensity Indication FAST blink (100 ms ON / 100 ms OFF, 5 Hz) → ABS actively modulating: HOLD brake command MEDIUM blink (200 ms ON / 200 ms OFF, 2.5 Hz) → Moderate ABS: pressure releasing SLOW blink (500 ms ON / 500 ms OFF, 1 Hz) → Light ABS activity: pressure reapplying OFF → ABS not active; normal braking or no braking Logic: Blink rate is mapped to computed slip ratio — higher slip = faster blink = stronger ABS modulation

## 2.2 GPIO and Peripheral Pin Assignment

| Signal | STM32 Pin | Direction | Peripheral Used | Notes |
| --- | --- | --- | --- | --- |
| Hall-effect Sensor INPUT | PA0 (TIM2_CH1) | INPUT | Timer 2 — Input Capture | Interrupt on rising edge; measures pulse period |
| ABS RED LED (LD5) | PD14 | OUTPUT | GPIO + TIM4 PWM | Toggle via timer interrupt for precise blink timing |
| Normal Brake LED (LD4) | PD12 | OUTPUT | GPIO | Simple digital output |
| Ready LED (LD3) | PD13 | OUTPUT | GPIO | System health indicator |
| Immobilizer LED (LD6) | PD15 | OUTPUT | GPIO | CAN-triggered output |
| CAN TX | PD1 (CAN1_TX) | OUTPUT | bxCAN1 | CAN bus transmit line |
| CAN RX | PD0 (CAN1_RX) | INPUT | bxCAN1 | CAN bus receive line |
| USART2 TX | PA2 | OUTPUT | USART2 | Debug output to ST-LINK virtual COM |
| USART2 RX | PA3 | INPUT | USART2 | Debug input (optional) |
| Joystick VRy — Brake Pedal | PA1 (ADC1_IN1) | INPUT (Analog) | ADC1 — Channel 1 | Y-axis analog voltage 0–3.3V; centre ~1.65V = no brake; full deflection = full brake; sampled at 200 Hz via ADC DMA |
| Joystick SW — Handbrake | PA3 (GPIO) | INPUT | EXTI3 | Push-down button; held >500 ms = handbrake active; sends 0x01 on CAN 0x300 |

Joystick VRy — Brake Pedal Mapping (ADC1, PA1, 12-bit: 0–4095 counts) Centre position (rest) ~1700–2400 counts → DEADZONE — no brake applied Light deflection 2400–3200 counts → Light brake (0–30% force); GREEN LED ON Moderate deflection 3200–3700 counts → Moderate brake (30–70% force); GREEN LED ON Full deflection 3700–4095 counts → Hard / panic brake (70–100%); ABS likely triggers ADC Setup: ADC1, single channel CH1 (PA1), 12-bit resolution, continuous mode with DMA. uint16_t adc_brake; // DMA fills this continuously; read in 5 ms control loop brake_pct = map(adc_brake, 2400, 4095, 0, 100); // 0–100% brake intensity

# 3. Sensor Selection — Hall-Effect Wheel Speed Sensor

## 3.1 Hall-Effect Sensor Types: Linear vs Digital (Analog vs Switch)

There is a third type — a Digital (Switch) Hall sensor — which is most commonly used for wheel speed sensing in ABS systems. The comparison below covers all three:

| Type | Output | Range | Typical Use | ABS Suitability |
| --- | --- | --- | --- | --- |
| Digital Hall Switch | Digital HIGH/LOW (GPIO compatible) | Binary; no amplitude info | Proximity, counting, RPM measurement | BEST — direct pulse counting for speed |
| Analog Hall Sensor | Analog voltage proportional to field strength | 0 V to Vcc continuously | Position sensing, current sensing | Usable — but requires comparator threshold logic |
| Linear Hall Sensor | Linear analog voltage; very precise | 0 V to Vcc; linear response | Precise position, current, torque measurement | Overkill — unnecessary precision; more complex |

RECOMMENDATION: Use a Digital Hall Switch (e.g., A3144, SS49E in switch mode, or AH3503) Why: Digital Hall sensors output a clean HIGH/LOW pulse for each magnet pass — exactly what a timer input capture needs. The STM32F407G TIM2 input capture on PA0 is designed for pulse-period measurement from a digital signal. A Linear Hall sensor outputs a slowly-varying analog voltage — you would need an external comparator (e.g., LM393) to convert it to a pulse, adding unnecessary hardware complexity. Recommended part: A3144 Hall Switch (5V) or TLE4905 (automotive grade, 5V) — both output clean digital pulses. Mount a small neodymium magnet on a rotating disc (wheel simulator) — one magnet = one pulse per revolution. For higher resolution: use 4, 8, or 16 magnets equally spaced on the disc (more pulses per revolution = better accuracy at low speed).

### 3.2 Physical Magnet and Sensor Mounting

The Hall-effect sensor requires small magnets physically attached to the rotating wheel disc to generate pulses. This is how real automotive wheel speed sensors work — the reluctor ring is replaced here by individual magnets. Follow the setup below exactly:

| Parameter | Detail |
| --- | --- |
| Number of magnets | 8 neodymium disc magnets (5 mm diameter × 2 mm thick) |
| Spacing on disc | Equally spaced at 45° intervals around the wheel face or rim |
| Magnet pole orientation | All magnets must face the sensor with the SAME pole (all North or all South facing out) — mixed poles cause missed or double pulses |
| Air gap (sensor to magnet) | 1 mm to 3 mm — critical; too large = sensor won't trigger; too small = magnet may strike sensor |
| Sensor mounting | Mount A3144 stationary on a fixed bracket; face pointing at the magnet path |
| Sensor output behaviour | A3144 is active-LOW open-collector — output goes LOW when magnet passes; HIGH otherwise |
| STM32 capture edge | Configure TIM2 Input Capture for FALLING edge (magnet present = LOW pulse) |
| Pulse per revolution | 8 (one per magnet) |
| Wheel stopped detection | If no falling edge received for > SENSOR_TIMEOUT_MS (500 ms), declare wheel stopped in firmware |
| Pull-up resistor | 10 kΩ from sensor output to 5V; then voltage divider (10kΩ + 3.3kΩ) to bring 5V signal down to ~3.3V for STM32 GPIO |

.

## 3.3 Wheel Speed Sensor Setup and Pulse Counting

| Parameter | Value / Method |
| --- | --- |
| Sensor Type | Digital Hall Switch |
| Operating Voltage | 5V (level-shift to 3.3V via 10kΩ + 3.3kΩ voltage divider before STM32 input) |
| Number of Magnets on Disc | 8 magnets (evenly spaced, 45° apart) — chosen for good low-speed resolution |
| Pulses per Revolution (PPR) | 8 |
| STM32 Input Pin | PA0 — TIM2 Channel 1 (Input Capture mode) |
| Capture Mode | Rising-edge capture; period = time between consecutive pulses |
| Timer Clock | 84 MHz (APB1 timer clock after PLL); Prescaler = 83 → 1 MHz timer tick |
| Timer Resolution | 1 µs per tick |
| Maximum measurable RPM | 1 µs period → 125,000 pulses/s → 15,625 RPS → theoretically unlimited; practical max: 8000 RPM |
| Minimum measurable RPM | ~0.5 RPM (timer overflow protection required; detect stopped wheel) |

# 4. Key Calculations and Mathematical Models

## 4.1 Wheel Speed and Vehicle Speed Calculation

Given wheel radius r = 1 inch = 25.4 mm = 0.0254 m

| Step | Formula | Example (at 1000 RPM) |
| --- | --- | --- |
| Pulse period (T_pulse) | T_pulse = time between rising edges (µs), measured by TIM2 | T_pulse = 7,500 µs at 1000 RPM with 8 magnets |
| Wheel RPM | RPM = (60 × 1,000,000) / (T_pulse × PPR) | = 60,000,000 / (7500 × 8) = 1000 RPM |
| Wheel RPS (rev per second) | RPS = RPM / 60 | = 1000 / 60 = 16.67 RPS |
| Wheel circumference (C) | C = 2π × r = 2 × 3.14159 × 0.0254 | = 0.15959 m per revolution |
| Wheel surface speed (V_wheel) | V_wheel = RPS × C (m/s) | = 16.67 × 0.15959 = 2.66 m/s |
| Vehicle speed (km/h) | V_kmh = V_wheel × 3.6 | = 2.66 × 3.6 = 9.58 km/h at 1000 RPM |
| Speed at max RPM (8000) | V_kmh = (8000/60) × 0.15959 × 3.6 | = 133.33 × 0.15959 × 3.6 = 76.6 km/h |

Note: The maximum speed of 76.6 km/h at 8000 RPM with a 1-inch radius wheel is intentionally low — this is a simulation/lab model. In a real vehicle with ~300 mm wheel radius, 8000 RPM would correspond to ~900 km/h. The small wheel radius is correct for your project prototype.

## 4.2 Slip Ratio Calculation (Core ABS Algorithm)

Slip ratio is the fundamental quantity that ABS monitors. It represents how much the wheel speed has dropped relative to the actual vehicle speed during braking.

| Parameter | Formula / Value |
| --- | --- |
| Reference Vehicle Speed (V_ref) | Estimated from the last known un-braked wheel speed; decays at –0.5 m/s² per iteration during braking (simulated) |
| Wheel Speed (V_wheel) | Measured in real-time from Hall sensor pulse period |
| Slip Ratio (λ) | λ = (V_ref – V_wheel) / V_ref × 100% |
| Normal braking (no ABS) | λ = 0–15% — wheel rolls freely, grip maintained |
| ABS trigger threshold | λ > 20% — wheel decelerating faster than vehicle, ABS activates |
| ABS target zone | λ = 15–25% — optimal friction coefficient range |
| Wheel lock (avoid this) | λ = 100% — wheel completely stopped, vehicle sliding |

## 4.3 LED Blink Rate Mapping from Slip Ratio

| Slip Ratio (λ) | ABS Phase | RED LED State | Blink Period | Interpretation |
| --- | --- | --- | --- | --- |
| 0 – 15% | No ABS — normal braking | OFF | — | GREEN LED ON (normal brake) |
| 15 – 25% | ABS HOLD phase | SLOW blink | 1000 ms (1 Hz) | Pressure held; monitoring slip |
| 25 – 40% | ABS RELEASE phase | MEDIUM blink | 400 ms (2.5 Hz) | Pressure releasing to recover wheel |
| 40 – 70% | ABS heavy modulation | FAST blink | 200 ms (5 Hz) | Strong ABS intervention |
| > 70% | Near wheel lock | VERY FAST blink | 100 ms (10 Hz) | Maximum ABS — hold brake command |

## 4.4 Distance Travelled During ABS Event

When ABS activates, the ECU tracks the cumulative distance using wheel speed (even though it differs from vehicle speed). This gives an approximation of distance travelled during the braking event.

| Step | Formula |
| --- | --- |
| Integration method | Euler integration: distance += V_wheel × Δt |
| Update interval (Δt) | Every 5 ms (200 Hz control loop) |
| Distance per loop | d_step = (RPS × C) × 0.005 metres |
| Cumulative ABS distance | D_ABS = sum of all d_step values from ABS start to ABS end |
| Reset condition | D_ABS resets to 0 when ABS deactivates (λ drops below 15%) |
| Transmission | D_ABS sent over CAN to Infotainment ECU (0x305 frame, every 100 ms) |
| Example at 1000 RPM | d_step = 16.67 × 0.15959 × 0.005 = 0.01329 m per loop step |

## 4.5 Brake Temperature Model (Imaginary / Simulated)

Since this is a simulation project, brake temperature is modelled mathematically — no physical temperature sensor is required. The model simulates heat generated by braking and cooling when brakes are released.

| Parameter | Formula / Value |
| --- | --- |
| Ambient temperature (T_ambient) | 30°C (constant; configurable via CAN command from Infotainment ECU) |
| Temperature rise per ABS cycle | ΔT_rise = slip_ratio × 0.5°C per 5 ms loop iteration |
| Temperature decay (cooling) | ΔT_cool = (T_brake – T_ambient) × 0.001°C per 5 ms loop (exponential cooling) |
| Heat model (each loop) | T_brake = T_brake + ΔT_rise – ΔT_cool |
| Normal braking heat rate | ΔT_rise = 0.1°C per 5 ms (lower slip) |
| Maximum simulated temperature | 250°C (upper clamp — brake fade threshold for display purposes) |
| Warning threshold | T_brake > 150°C → send warning flag over CAN to Infotainment ECU |
| Critical threshold | T_brake > 200°C → ORANGE LED (LD3) blinks slowly to indicate brake thermal warning |

## 4.6 RPM Cross-Check with Engine ECU

The Engine ECU transmits engine RPM over CAN (CAN ID 0x101, every 50 ms). The ABS ECU uses this to:

- Cross-check wheel speed: at gear ratio 1:1, wheel RPM ≈ engine RPM (simplified assumption for prototype)

- Detect unreasonable discrepancy: if engine RPM > 500 but wheel speed = 0 for > 500 ms → possible sensor fault or wheel lock

- Maximum RPM reference: engine RPM is clamped to 8000 RPM max; above this, Engine ECU sends a throttle-cut request

| Engine RPM | Expected Wheel Speed (at 1:1 ratio) | ABS Action |
| --- | --- | --- |
| 0 – 500 RPM | < 5 km/h | ABS system in standby; minimum speed not met |
| 500 – 2000 RPM | 5 – 15 km/h | ABS active range; low speed mode |
| 2000 – 5000 RPM | 15 – 38 km/h | Full ABS operation range |
| 5000 – 8000 RPM | 38 – 76 km/h | High speed; ABS most effective |
| > 8000 RPM (fault) | Out of range | Engine ECU sends throttle-cut; ABS logs fault |

# 5. ABS Control Algorithm Design

## 5.1 Control Loop Overview.

| Loop Step | Action | Peripheral Used |
| --- | --- | --- |
| 1. Read wheel speed | Get pulse period from TIM2 input capture; compute wheel RPM and V_wheel | TIM2 — Input Capture |
| 2. Update V_ref | Decay vehicle reference speed if braking; hold if no brake | Software model |
| 3. Compute slip ratio | λ = (V_ref – V_wheel) / V_ref × 100% | FPU arithmetic |
| 4. ABS state machine | Evaluate slip against thresholds; determine HOLD / RELEASE / APPLY state | Software FSM |
| 5. LED control | Set RED LED blink timer period based on ABS state; set GREEN/ORANGE as needed | TIM4 — PWM / GPIO |
| 6. Update temperature | Apply heating/cooling model; check thresholds | Software model |
| 7. Update distance | Accumulate d_step if ABS active | Software accumulator |
| 8. CAN TX queue | Package data into CAN frames; pass to CAN TX FIFO | bxCAN1 |
| 9. CAN RX check | Process any received CAN messages (RPM, immobilizer, commands) | bxCAN1 RX FIFO |

## 5.2 ABS Finite State Machine (FSM)

| State | Entry Condition | Actions | Exit Condition |
| --- | --- | --- | --- |
| STANDBY | V_wheel < 5 km/h OR no brake input | All LEDs idle; ABS disabled; CAN heartbeat only | V_wheel > 5 km/h AND brake pressed |
| NORMAL_BRAKE | Brake pressed; λ < 20% | GREEN LED ON; normal brake force; track speed and temp | λ > 20% → ABS_HOLD |
| ABS_HOLD | λ > 20% | RED LED SLOW blink; hold inlet valve closed; start distance counter | λ > 30% → ABS_RELEASE; λ < 15% → NORMAL_BRAKE |
| ABS_RELEASE | λ > 30% | RED LED MEDIUM blink; release pressure; wheel accelerates | Wheel speed recovers; λ < 25% → ABS_REAPPLY |
| ABS_REAPPLY | λ < 25% after release | RED LED FAST blink; gradually reapply pressure | λ < 15% → NORMAL_BRAKE; λ > 35% → ABS_RELEASE |
| EMERGENCY_BRAKE | Immobilizer CAN message received | BLUE LED ON; all others OFF; full brake applied; ABS bypassed | Immobilizer signal cleared |
| FAULT | Sensor timeout / CRC error / overvoltage | ORANGE LED blinks; ABS disabled; fault DTC logged; CAN fault broadcast | Manual reset or power cycle |

## 5.3 Immobilizer Integration (ADAS ECU)

When the ADAS / Immobilizer ECU (CAN ID 0x800) detects an obstacle or security event, it broadcasts an immobilization command on CAN ID 0x801. The ABS ECU must:

- Receive and validate the 0x801 CAN frame within one loop cycle (5 ms)

- Immediately transition to EMERGENCY_BRAKE state regardless of current state

- Apply maximum simulated brake force (set all ABS valves to maximum hold)

- Disable ABS modulation — full lock is acceptable in an emergency stop

- Illuminate BLUE LED (LD6) solid; extinguish all other LEDs

- Broadcast ABS status 0xFF (emergency) on CAN 0x301 to alert all other ECUs

- Maintain emergency brake until a CLEAR command (0x800, byte 0 = 0x00) is received

# 6. CAN Bus Communication Design

## 6.1 CAN Bus Configuration

| Parameter | Value |
| --- | --- |
| CAN Standard | CAN 2.0B (Extended and Standard frames) |
| Bus Speed | 500 kbps |
| STM32 CAN Peripheral | bxCAN1 (CAN1) |
| CAN TX Pin | PD1 (AF9 — CAN1_TX) |
| CAN RX Pin | PD0 (AF9 — CAN1_RX) |
| CAN Transceiver | TJA1050 or MCP2551 (external IC required; 5V; connect to PD0/PD1 via 3.3V-compatible transceiver) |
| Termination | 120 Ω at each end of the bus (two resistors total for the network) |
| Frame Type | Standard 11-bit CAN ID |
| TX Mode | Automatic retransmission; no loopback |
| RX Filters | Hardware acceptance filter set to pass only relevant IDs for this node |
| CAN Timing (168 MHz clock) | Prescaler=12; BS1=8; BS2=3 → 500 kbps (verify with CAN calculator) |

## 6.2 CAN Message Map — ABS ECU Transmits

| CAN ID | Frame Name | Byte Layout | Frequency | Recipients |
| --- | --- | --- | --- | --- |
| 0x300 | ABS_HEARTBEAT | B0: System state (0=OK,1=ABS active,0xFF=Emergency); B1: LED state bitmask | 100 ms | All ECUs |
| 0x301 | ABS_WHEEL_SPEED | B0-B1: V_wheel in km/h (×10, uint16); B2-B3: Wheel RPM (uint16); B4: Sensor status | 50 ms | Engine, Infotainment, Telematics |
| 0x302 | ABS_SLIP_STATUS | B0: Slip ratio % (uint8, 0–100); B1: ABS State (FSM state enum); B2: Blink rate code | 50 ms | Infotainment, Telematics |
| 0x303 | ABS_TEMPERATURE | B0-B1: Brake temperature °C (uint16); B2: Temp warning flag (0/1/2) | 200 ms | Infotainment, BMS, Telematics |
| 0x304 | ABS_DISTANCE | B0-B3: ABS event distance (mm, uint32); B4-B7: Total odometer (mm, uint32) | 100 ms | Infotainment, Telematics |
| 0x305 | ABS_FAULT | B0: DTC code; B1: Fault source; B2-B7: Freeze-frame data snapshot | On event only | All ECUs (broadcast) |

## 6.3 CAN Message Map — ABS ECU Receives

| CAN ID | Source ECU | Content | ABS Action |
| --- | --- | --- | --- |
| NA | Engine ECU | Engine RPM (uint16, bytes 0-1) | Cross-check wheel speed; detect RPM limit breach |
| NA | Transmission ECU | Current gear (uint8, byte 0); gear ratio (float encoded) | Refine V_ref calculation if gear ratio available |
| NA | Infotainment ECU | Ambient temperature setting (byte 0 in °C) | Update T_ambient in brake temperature model |
| NA | ADAS/Immobilizer ECU | Immobilize command (byte 0: 0x01=immobilize, 0x00=clear) | Trigger EMERGENCY_BRAKE state |
| NA | Airbag / SRS ECU | Crash impact signal (byte 0: 0x01=impact detected, 0x00=clear) | Log crash event as DTC freeze-frame; transition to FAULT state; broadcast fault on 0x305 |
| NA | BMS ECU | Battery voltage (uint16, ×100 mV, bytes 0-1) | Log for fault correlation; no direct ABS action |
| NA | Telematics ECU | Remote diagnostic request (byte 0: 0x10=send all data) | Respond with full status dump on 0x305 |

# 7. Software Architecture

## 7.1 Firmware Structure

| Module / File | Responsibility | Language |
| --- | --- | --- |
| main.c | System init; peripheral setup; main loop; CAN init | C (HAL) |
| abs_fsm.c / .h | ABS Finite State Machine; state transitions; ABS phase logic | C |
| wheel_speed.c / .h | TIM2 input capture ISR; pulse period to RPM/speed conversion | C |
| led_control.c / .h | LED blink patterns; TIM4 interrupt-driven blink rate control | C |
| can_handler.c / .h | CAN TX message assembly; CAN RX parsing; filter config | C |
| brake_temp.c / .h | Temperature model; heating/cooling equations; threshold check | C |
| distance_calc.c / .h | Distance accumulator; reset logic; odometer tracking | C |
| fault_manager.c / .h | DTC storage; plausibility checks; fault state handling | C |
| config.h | All #define constants: thresholds, timing, CAN IDs, wheel params | C Header |

## 7.2 Interrupt Priority Assignment

| Interrupt | Priority | Reason |
| --- | --- | --- |
| TIM2 — Input Capture (Hall sensor) | 0 (Highest) | Wheel speed measurement is time-critical; cannot be delayed |
| TIM6 — ABS Control Loop (5 ms) | 1 | Control loop must run deterministically after sensor update |
| CAN1 RX FIFO0 — Message received | 2 | Immobilizer command must be processed within one loop cycle |
| TIM4 — LED blink timer | 3 | LED update; low latency needed for accurate blink rate |
| CAN1 TX — Transmit complete | 4 | Non-critical; just clears TX flag |
| USART2 — Debug output | 5 (Lowest) | Debug; never interferes with real-time functions |

## 7.3 Key Configuration Constants (config.h)

| #define Constant | Value | Meaning |
| --- | --- | --- |
| JOYSTICK_DEADZONE_LOW | 1700 | ADC count below which brake is not registered (centre deadzone lower bound) |
| JOYSTICK_DEADZONE_HIGH | 2400 | ADC count above which brake input begins (centre deadzone upper bound) |
| JOYSTICK_FULL_BRAKE | 3700 | ADC count above which hard/panic braking is assumed |
| HANDBRAKE_HOLD_MS | 500 | Duration SW button must be held to confirm handbrake (ms) |
| WHEEL_RADIUS_M | 0.0254f | Wheel radius in metres (1 inch) |
| MAGNETS_PER_REV | 8 | Number of Hall-effect trigger magnets on wheel disc |
| CONTROL_LOOP_MS | 5 | ABS control loop period in milliseconds |
| ABS_TRIGGER_SLIP | 20 | Slip ratio % at which ABS activates |
| ABS_TARGET_SLIP_HI | 25 | Upper slip threshold (enter release phase) |
| ABS_TARGET_SLIP_LO | 15 | Lower slip threshold (ABS deactivates) |
| MAX_ENGINE_RPM | 8000 | Maximum engine RPM from Engine ECU |
| TEMP_AMBIENT_DEFAULT | 30 | Default ambient temperature in °C |
| TEMP_WARN_LEVEL1 | 150 | First brake temperature warning threshold (°C) |
| TEMP_WARN_LEVEL2 | 200 | Critical brake temperature threshold (°C) |
| BLINK_FAST_MS | 100 | RED LED ON+OFF period for maximum ABS (ms) |
| BLINK_MEDIUM_MS | 400 | RED LED period for moderate ABS (ms) |
| BLINK_SLOW_MS | 1000 | RED LED period for light ABS hold (ms) |
| CAN_TX_SPEED_PERIOD | 50 | CAN wheel speed message transmit period (ms) |
| CAN_TX_STATUS_PERIOD | 100 | CAN ABS status message transmit period (ms) |
| CAN_TX_TEMP_PERIOD | 200 | CAN temperature message transmit period (ms) |
| SENSOR_TIMEOUT_MS | 500 | Max time without pulse before wheel declared stopped |